Annexure IV conted…

 B – The Schedule (Part of Annexure IV)
	Item No.
	Quantity
	Description
	Rate

(Rs.)
	Unit
	Amount

(Rs.)

	1.
	100 Sq Mt
	Repairing the damaged position of plaster by providing washed stone grit plaster, rendering with cement mortar in 1:4 (one cement and four sand) as per existing design/pattern and thickness with Neeru finish/sand faced finish in patches to match with the neighboring surfaces including carefully braking the existing plaster at all levels and floors and carting away the debris, curing, scaffolding etc. complete as directed. 
	
	LS.
	

	2.
	4608.7 Sq. Mt.
	Providing and applying base coat of oil based cement primer of approved manufacture and two or more coats oil bound distemper of approved manufacture and in desired shades and finish (plain or roller finish) including making the surfaces, applying putty on surfaces to make the surfaces even to the interior surfaces of walls, ceiling, beams, canopies, staircase, lobbies etc. including scaffolding etc. Complete as directed (Bed Rooms). 
	
	Sq. Mt.
	

	3.
	1242.6 Sq. Mt.
	Preparing the surface smooth by scraping sand-papering and filling the unevenness and undulation with putty and P.O.P., providing and applying one coat of cement primer and thereafter providing and applying two or more coats of 1st quality plastic emulsion paint of approved make on walls/ceilings in desired shades and finish (plain or roller finish) including scaffolding etc. complete all as directed by Bank in charge (Drawing Room).


	
	Sq. Mt.
	

	4.
	5370 Sq. Mt.
	Providing and applying two or more coats or 1st quality synthetic enamel paint /French sprit polish shade to teakwood/commercial board surface in doors, windows, shelves loft, wooden cabinet, Wardrobes, drawers(inside also) Kitchen cabinet, pelmets etc. of approved manufacture as per specification including scaffolding etc. Complete as directed.
	
	Sq. Mt.
	

	5
	68.4 Sq. Mt.
	Washed stone grit plaster on exterior walls of any height. Above level in two layers, under layer 12 mm cement plaster 1:4 (1 cement 4 coarse sand) furrowing the under layer with scratching tool, applying cement slurry on the under layer @ 2 Kg of cement per square meter, top layer 15mm cement plaster 1:1/2:2 (1 cement :1/2 coarse sand:2 stone chipping 10mm nominal size) in panels with groove all around as per approved pattern including scrubbing and washing, the top layer with brushes and water to expose the stone chippings, complete. (Exterior quality) of approved manufacture and shade to walls etc. as per specification and direction of engineer-in-charge. 
	
	Sq. Mt.
	

	6
	100 Nos.

60 Nos.
	P/A two or more coats of 1st quality synthetic enamel paint of approved make & shade on ceiling fan/exhaust fan as an entire unit including properly cleaning, sand papering & applying putty to make the surface even before painting including carefully taking out the existing exhaust fan (from frame) and ceiling fan with caps & rods from fan hook, installation of fan after painting & making electric connection etc. all as directed.

(a) Ceiling fan

(b) Exhaust fan 
	
	No

No
	

	7.
	Job Work for 20 flats
	Applying two coats or more of synthetic enamel paint/ flat oil paint/Snowcem Plus/varnishing as required to GI pipes, fan hooks, bracket of wash basins and sinks, switchboards and meter boards etc. in all flats, exposed CI/AC soil and rain water pipes manhole covers, MS covers, hand rails of all stairs,  electric conduit pipes,  angle iron post, fire hydrant posts including scrapping, scaffolding etc. complete as directed
	
	L.S. 
	

	
	
	Total


	
	
	


Place ……………..

Date ……………...

                           Signature, address 
& Tel no of the contractor with seal   
